SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE: CSA0713                            COURSE NAME: COMPUTERNETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I Suganthan .K-(192424105) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

K.Suganthan

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the

CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission

link. What is the result of the receiver&#39;s CRC calculation? How does the receiver know that an error has occurred?

=

Given:

- Message = 11100011

- Generator Polynomial = x³ + 1

- Generator Binary = 1001

(i) Determine the transmitted message

Since the generator polynomial has degree 3, append three zeros to the original message.

Original Message:
11100011

Message after appending zeros:
11100011000

Perform modulo-2 (XOR) division of 11100011000 by the generator 1001. The remainder obtained is:

CRC Remainder = 010

Append the remainder to the original message.

Transmitted Message = 11100011010

(ii) Error Detection

Assume the leftmost bit is inverted during transmission.

Received Message = 01100011010

The receiver performs the same CRC division using the generator 1001.

The division produces a non-zero remainder (110).

Since the remainder is not equal to 000, the receiver concludes that an error has occurred during transmission. Therefore, the received frame is considered corrupted and is rejected (or retransmission is requested if the protocol supports it).

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per

minute with each frame carrying an average of 10,000 bits. What is the throughput of this

network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data

communications. The signal- to-noise ratio is 3162. Find the channel capacity

=

2(a). Throughput Calculation

Given:

- Bandwidth = 10 Mbps

- Number of frames transmitted = 12,000 frames/minute

- Size of each frame = 10,000 bits

Formula

Throughput = (Frames × Bits per Frame) / Time

Calculation

Total bits transmitted per minute

= 12,000 × 10,000

= 120,000,000 bits

Convert into bits per second

= 120,000,000 ÷ 60

= 2,000,000 bits/second

Answer

Throughput = 2 Mbps

2(b). Channel Capacity

Given:

- Bandwidth (B) = 3000 Hz

- Signal-to-Noise Ratio (S/N) = 3162

Formula (Shannon Capacity)

C = B log₂(1 + S/N)

Calculation

C = 3000 × log₂(3163)

≈ 3000 × 11.63

≈ 34,890 bits/second

Answer

Channel Capacity ≈ 34.9 kbps

3. A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit

patients&#39; heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

=

3. Hamming (12,8) Error-Correcting Code

The hospital uses the Hamming (12,8) error-correcting code to ensure accurate transmission of patient health data over a wireless network.

In Hamming (12,8), 8 bits are used for data and 4 bits are used for parity, making a total of 12 bits.

The sender places the data bits in positions 3, 5, 6, 7, 9, 10, 11, and 12, while the parity bits are placed in positions 1, 2, 4, and 8. The parity bits are calculated using the even parity method.

During wireless transmission, electromagnetic interference may change one bit. At the receiver, the parity bits are recalculated to produce a syndrome value. The syndrome identifies the exact position of the erroneous bit. The receiver then flips that bit to correct the error and recovers the original patient data without requesting retransmission.

Advantages

- Detects and corrects one-bit errors.

- No retransmission is required.

- Provides reliable patient monitoring.

- Reduces communication delay.

- Ensures accurate medical data is delivered to doctors.

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a

/27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting

techniques to calculate the total number of subnets created and the number of usable hosts per

subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the

network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address

2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the

given address into its shortest form and then expand the compressed address back to its full

notation. Using a prefix length of /64, determine the network prefix of the given address.

Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

=

4(a). IPv4 Subnetting

Given:

- Network Address = 172.16.10.0/24

- Subnet Mask = /27

Step 1: Number of Subnets

Borrowed bits = 27 − 24 = 3

Number of subnets

= 2³

= 8 subnets

Step 2: Number of Usable Hosts

Host bits

= 32 − 27

= 5

Total addresses

= 2⁵

= 32

Usable hosts

= 32 − 2

= 30 usable hosts per subnet

Step 3: Subnet Ranges

| Subnet | Address Range |
| --- | --- |
| 172.16.10.0 | 0 – 31 |
| 172.16.10.32 | 32 – 63 |
| 172.16.10.64 | 64 – 95 |
| 172.16.10.96 | 96 – 127 |
| 172.16.10.128 | 128 – 159 |
| 172.16.10.160 | 160 – 191 |
| 172.16.10.192 | 192 – 223 |
| 172.16.10.224 | 224 – 255 |

Step 4: Identify the Subnet

The IP address 172.16.10.78 falls within the range 64–95.

Therefore,

- Network Address: 172.16.10.64

- Broadcast Address: 172.16.10.95

The IP address 172.16.10.95 is the broadcast address of the same subnet. Hence, it belongs to the same subnet but cannot be assigned to any host.

4(b). IPv6 Addressing

Given IPv6 Address

2001:0db8:0000:0000:0000:ff00:0042:8329

Compressed Form

2001:db8::ff00:42:8329

Expanded Form

2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix (/64)

Using a /64 prefix, the first 64 bits represent the network.

Network Prefix = 2001:db8::/64

Number of Host Addresses

Host bits

= 128 − 64

= 64

Possible host addresses

= 2⁶⁴

= 18,446,744,073,709,551,616 host addresses

5. A university is conducting an online semester examination for 8,000 students

simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted

question papers to all students. The university server is connected through a 10 Gbps high-

speed network, while students access the examination portal using different internet

connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile

networks (5–10 Mbps). During the first few minutes of the examination, many students with

slower internet connections report incomplete downloads, repeated retransmissions, delayed

acknowledgments, and temporary freezing of the examination portal. Network administrators

observe that the server continues transmitting data at a high rate even though several student

devices are unable to receive and process packets at the same speed. This results in receiver

buffer overflow, unnecessary retransmissions, increased network traffic, and delays in

delivering the question papers. Analyze the above scenario and identify the primary

networking problem. Explain how the mismatch between the sender&#39;s transmission speed and the receiver&#39;s processing capability affects communication.

=

5. University Online Examination Scenario

Primary Networking Problem

The primary networking problem in this scenario is Flow Control.

Explanation

The university examination server is connected through a 10 Gbps network and transmits encrypted question papers at a very high speed. However, students receive the data through different types of internet connections, such as 100 Mbps fiber broadband, 20 Mbps home Wi-Fi, and 5–10 Mbps mobile networks.

Because the server sends data much faster than many student devices can receive and process it, the receivers are unable to handle all incoming packets. This causes the receiver's buffer memory to overflow, resulting in packet loss.

The lost packets must be retransmitted, increasing network traffic and causing delays. Delayed acknowledgments and repeated retransmissions further reduce the efficiency of the communication. As a result, some students experience incomplete downloads and temporary freezing of the examination portal.

To solve this problem, flow control mechanisms, such as the TCP Sliding Window protocol, regulate the sender's transmission rate according to the receiver's available buffer space and processing capability. This prevents buffer overflow, reduces retransmissions, improves network efficiency, and ensures reliable delivery of the examination papers.

Conclusion

The communication problem is caused by the mismatch between the sender's high transmission speed and the receiver's lower processing capability. Flow control ensures that the sender transmits data only at a rate that the receiver can handle, thereby preventing packet loss, minimizing retransmissions, and ensuring smooth and reliable communication.